4K HDMI Audio Extractor With Up/Down Scaler Series

ITEM NO.: HAE01 4K HDMI Audio Extractor With Up/Down Scaler  1 in 2 out

The HAE01 is an HDMI multifunction tool: allow you to extract 4K HDMI for both digital optical S/PDIF (Toslink) and stereo analaog audio signal for use by an amplifier or AV receiver.  With a special feature to do upscale 1080p signal up to 4K or Downscale 4K signal to 1080p.  Full OSD operation by using 5 way joystick switches for easy set up and doing EDID management.  It is a perfect solution for integrating a 4K source into a 1080P HD system or vice versa.

HAE01 4K HDMI Audio Extractor with Up/Down Scaler 1 in 2 out

- Support HDMI 2.0, HDCP 2.2, 4K@60Hz 4:4:4 18Gbps（6Gbps per color）3D,HDR.

- Support extracts the 4K HDMI audio and output to a multi-channel digital or analog audio signal.

- Allows you to route HDMI audio simultaneously via S/PDIF and 3.5mm stereo audio outputs.

- HDMI audio output:  LPCM7.1@192KHz, Dolby TrueHD, DTS-HD MA.

- Input:  HDMI x 1, Output: HDMI x 2, Optical out x 1, analog stereo out x 1.

- Support uncompressed audio such as LPCM.

- Support 1080p to 4K upscale and 4K to 1080p downscale.

- Support 4K 4:4:4 convert to 4:2:0.

- Audio output support: up to 7.1 channel

- Support CEC pass-through

- Built in EDID management, 2/5.1/7.1 patched EDID.

- Built in 5 way joystick switch for full OSD operation.

5 way Joystick Switch Function:

MENU Operation

| SW1 | Function |
| --- | --- |
|  | Up |
|  | Down |
|  | Left |
|  | Right |
|  | confirm |

MENU :

| Main Menu | Sub Menu | Sub Menu Function |
| --- | --- | --- |
| OUTPUT 1 MODE | NORMAL | Output video same as input Video |
|  | UPSCALE | 1080P Upscale to 4K |
|  | EXIT | Exit Sub Menu |
| OUTPUT 1 SUBSAMPLING | NORMAL | Output color subsampling same as Input |
|  | 4:4:4 4:2:0 | 4K 4:4:4 convert to 4K 4:2:0 |
|  | NORMAL | Output color subsampling same as Input |
| OUTPUT 1 AUDIO MODE | OUTPUT 1 AUDIO MODE | Audio Selection |
|  | NORMAL | audio output |
|  | MUTE | No audio output |
|  | EXIT | Exit Sub Menu |
| OUTPUT 2 MODE | AUDIO ONLY | Audio output only |
|  | DOWNSCALE | 4K downscale to 1080P |
|  | SPLITTER | Distribution Mode（Output signal same as OUTPUT 1） |
|  | EXIT | Exit Sub Menu |
| EDID(VIDEO) | AUTO | Use OUTPUT 1 EDID as priority, if OUTPUT 1 and OUTPUT 2 not connected with TV, Default as 4K |
|  | OUTPUT 1 | Use OUTPUT 1 EDID |
|  | OUTPUT 2 | Use OUTPUT 2 EDID |
|  | PRESET 4K | Default 4K 60Hz 8bit |
|  | PRESET 1080P | Default 1080P 60Hz 8bit |
|  | EXIT | Exit Sub Menu |
| EDID(AUDIO) | AUTO | Use OUTPUT 2 as priority, If OUTPUT 1 and OUTPUT 2 not connected with TV, Default as LPCM 2.0 |
|  | OUTPUT 1 | Use OUTPUT 1 EDID |
|  | OUTPUT 2 | Use OUTPUT 2 EDID |
|  | 5.1 AUDIO | Default LPCM 5.1 |
|  | 2.0 AUDIO | Default LPCM 2.0 |
|  | EXIT | Ext Sub Menu |
| EXIT & SAVE |  | Save setting and Exit MENU |

Panel View:

LED Indication:

| Color | LED |
| --- | --- |
| GREEN | Power |
| BLUE | Output 1 |
| RED | Output 2 |

RS232 Define:

DB9(F)/3.5mm STEREO PHONE JACK

DB9(F)/3.5mm STEREO PHONE JACK

| RS232 |  | 3.5 mm Phone Jack |  |
| --- | --- | --- | --- |
| Footprint | Define | Footprint | Define |
| Pin 2 | TX | Pin 1 | RX |
| Pin 3 | RX | Pin 2 | TX |
| Pin 5 | GND | Pin 3 | GND |

Trouble Shooting:

- CEC function only works Output 1.

- Any HDMI output with HDCP2.2 compatible issues will cause HDMI no video output,  Under HDCP 2.2 mode: any one monitor HDCP authorization failed will cause no video status, other connecting monitors will be also failed.  Then monitor testing must be one by

- Up/down scale mode only support resolution conversion, frequency will be  not changed.

4. Under HDMI 2.0 mode, must use Premium certified HDMI cable.

HAE01 Package Include:

HAE01 x  1 pcs

DC5V 1Amp Power Adapter x 1 pcs

DB9(F)/3.5mm STEREO PHONE JACK	 x 1 pcs

Mounting Screw pack x 2 bags

Specification:

| ITEM NO | HAE01 |
| --- | --- |
| HDMI Input | HDMI Type A x 1 |
| HDMI Output | HDMI Type A x 2 |
| HDMI Video Support | Up to 4096x2160/60Hz/4:4:4(18Gbps) 、 3D、HDR |
| HDMI Audio Support | Up to 7.1 LPCM 192KHz / Dolby TrueHD / DTS-HD MA |
| Digital Audio Connector | TOSLINK Type x 1 |
| Analog Audio Connector | 3.5mm Phone Jack x 1 |
| Firmware updated Interface | DB9(F)/3.5mm STEREO PHONE JACK |
| Power Supply | Regulated DC5V / 1A |
| Power Consumption | 4W |
| Temperature | Operation：-10~45℃, Storage：-30~70℃,Humidity：up to 95% |
| Dimension mm |  |
| Weight g |  |

Rev. A