4x4 Seamless Multiviewer/Matrix/Videowall

ITEM NO.: HM44

HM44 4x4 HDMI Seamless Matrix is the perfect solution for displaying content from up to four sources on four HDTV displays. Each of the sources can be individually scaled. Seamless switching ensures no frame loss during picture transitions. Eight preset buttons allow the user to store and recall custom-configured routing presets. Build in OSD menu simplifies system configuration. The HM44 HDMI Seamless Matrix is a great solution for small and large commercial venues that require content from four sources displayed on four displays instantly and without any loss of picture.

HM44  4x4 Seamless Multiviewer/Matrix/Videowall

- Input and Output resolutions up to 1080p Full HD and 1920x1200 (WUXGA)

- Support LPCM7.1 @ 192KHz, Dolby True HD, DTS-HD MA

- HDMI1.3, DVI1.0, and HDCP 1.4 compliant

- Video-Wall mode supports 2 x 2 Video-Wall with 4 selectable input sources

- Support 4 input and 4 multiview windows seamless switching

- Multiview mode support scales and sizes each picture.

- Multiview mode support PIP, POP and set PIP priority

- Support panel button, IR remote, RS232, Web GUI and APP control

- Support OSD menu setting

- Field-upgradable firmware for both web server and RS232.

Application View:

Panel View ：

Front Panel Function:

1

3

6

5

2

4

Panel Button Function:

| No. | Function Name | Description |
| --- | --- | --- |
| 1 | IR | Receiving IR remote signal |
| 2 | POWER | Power on / standby |
| 3 | INPUT 1~4 | Press these buttons to select the desired input |
| 4 | OUTPUT 1~4 | Press these buttons to select the desired output (not available for videowall mode) |
| 5 | MULTIVIEW | Enter Multiview mode (All output display same screen) |
|  | MATRIX | Enter Matrix mode |
|  | VIDEOWALL | Enter videowall mode |
|  | LOCK | Use this button to lock / unlock panel button and prevent accidental changes. |
|  | MENU | - Enter OSD menu - Back to upper level of menu |
|  | ALL | Select all output (not available for videowall mode ) |
|  | OFF | Turn off the selected output (not available for videowall mode ) |
|  | LOAD | - Load layout/source in Multiview and Matrix mode from scene 1~8 - Load screen position in VIDEOWALL mode from scene 1~4 |
|  | SAVE | - Save current layout/source in Multiview and Matrix mode to scene 1~8 - Save screen position in VIDEOWALL mode to scene 2~4 |
|  | FUNCTION | - Switch different PIP/POP layout in Multiview Mode - Bezel correction (adjust screen position) in VIDEOWALL MODE - Not available in MATRIX MODE |
| 6 | UP | - Move menu function up in OSD - Move screen position up in videowall mode. |
|  | DOWN | - Move menu function down in OSD - Move screen position down in videowall mode. |
|  | LEFT | - Move menu function left in OSD - Move screen position left in videowall mode. |
|  | RIGHT | - Move menu function right in OSD - Move screen position right in videowall mode. |
|  | ENTER | Execution selected function |

Operating:

Multiview mode: Output/input selection: - Press OUTPUT 1~4 buttons to select the desired output windows (A~D), button LED will light on. You could select multiple outputs at the same time. - Press INPUT 1~4 buttons to switch to desired input - Press ENTER to quit the input selection. Window layout selection: - Press FUNCTION button to enter layout selection. - Press INPUT /OUTPUT 1~4 to select desired layout, total 8 sets. - Press ENTER to quit the layout selection. Save scene: - Press SAVE button. - Press INPUT /OUTPUT 1~4 to save current windows layout and source of windows. - Press ENTER to confirm. Load scene: - Press LOAD button. - Press INPUT /OUTPUT 1~4 to load saved layout/source. - Press ENTER to confirm.

Matrix mode: Output/input selection: - Press OUTPUT 1~4 buttons to select the desired output monitor, button LED will light on. You could select multiple OUTPUT at the same time. - Press INPUT 1~4 buttons to switch to desired input - Press ENTER to quit the input selection. Save scene: - Press SAVE button. - Press INPUT /OUTPUT 1~4 to save current source of output. - Press ENTER to confirm. Load scene: - Press LOAD button. - Press INPUT /OUTPUT 1~4 to load saved output/source, total 8 sets. - Press ENTER to confirm.

Videowall mode: Input selection: - Press INPUT 1~4 buttons to switch to desired input Bezel correction: - Press FUNCTION button. - Press OUTPUT 1~4 to select the monitor to adjust screen position, shift depends on output resolution. - Press INPUT 1~4 to select the monitor to adjust screen position, shift depends on input resolution. - Press UP, DOWN, LEFT or RIGHT button to shift the screen. - Press ENTER to quit the position adjustment. Save scene: - Press SAVE button. - Press INPUT 2~4 to save current position of each screen. - Press ENTER to confirm. Load scene: - Press LOAD button. - Press INPUT 1~4 to load saved position of each screen(1 is factory default), total 4 sets. - Press ENTER to confirm.

Rear Panel Function:

1

2

4

6

3

5

Panel Connector Function:

| No. | Name | Description |
| --- | --- | --- |
| 1 | INPUT 1~4 | HDMI Input 1~4 |
| 2 | OUTPUT 1~4 | HDMI Output 1~4 |
| 3 | ETHERNET | Yellow LED: Link Green LED: Data transmit/receive |
|  |  | WebGUI control / Web firmware update |
| 4 | RS232 | RS232 control / firmware update |
| 5 | DC12V | External power supply |
| 6 | I/O | I: Power on O: Power off |

IR Remote Control (IR01) :

Initial at first time use the remote control or after change battery of remote control, the IR remote control and the equipment IR Remote ID must be using same. The default IR Remote ID is 0.

To setting the Remote ID, Press and hold power button, then press button 0 to complete the setting. +.(for example)

Remote Control Button Function:

| Symbol | Function Name | Description |
| --- | --- | --- |
|  | POWER | Power on / standby |
|  | UP | Move menu function up in OSD |
|  | DOWN | Move menu function down in OSD |
|  | LEFT | Move menu function left in OSD |
|  | RIGHT | Move menu function right in OSD |
|  | ENTER | Execution selected function |
|  | MENU | - Enter OSD menu - Back to upper level of menu |
|  | Quick output/input selection | - Press to select output 1 - Switch to input 1 if output have been selected |
|  | Quick output/input selection | - Press to select output 2 - Switch to input 2 if output have been selected |
|  | Quick output/input selection | - Press to select output 3 - Switch to input 3 if output have been selected |
|  | Quick output/input selection | - Press to select output 4 - Switch to input 4 if output have been selected |
|  | Reserved |  |
|  | Reserved |  |
|  | Multiview | Switch to Multiview Mode |
|  | Matrix | Switch to Matrix mode |
|  | Videowall | Switch to Videowall mode |
|  | Turn off video | Turn off video if output have been selected |
|  | Reserved |  |
|  | Reserved |  |
|  | Reserved |  |
|  | Reserved |  |

OSD MENU :

| MAIN | SUB MENU | DESCRIPTION |  |  |
| --- | --- | --- | --- | --- |
| SETUP | OUTPUT RESULTION | 480P 1366x768 576P 1440x900 720P50 1600x1200 720P60 1680x1050 1080P24 1920x1200 1080P50 BACK 1080P60, Default EXIT 1024x768 1280x800 1280x1024 |  |  |
|  | PICTURE SETTING | OUT SELECT | OUT1~OUT4, Default OUT1 |  |
|  |  | CONTRAST | 0~100, Default 50 |  |
|  |  | BRIGHTNESS | 0~100, Default 50 |  |
|  |  | SATURATION | 0~100, Default 50 |  |
|  |  | HUE | 0~100, Default 50 |  |
|  |  | RESET |  |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | OSD SETTING | MENU TIMEOUT | 0~100, Default 20 |  |
|  |  | INFO TIMEOUT | 0~100, Default 5 |  |
|  |  | INFO POSITION | RIGHT TOP |  |
|  |  |  | LEFT TOP , Default |  |
|  |  |  | RIGHT BUTTON |  |
|  |  |  | LEFT BUTTON |  |
|  |  | INFO DISPLAY | ON, OFF, Default ON |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | EDID | INPUT1 EDID | 1080P 2CH, Default |  |
|  |  |  | 1080P MLT |  |
|  |  |  | OUTPUT 1 |  |
|  |  |  | OUTPUT 2 |  |
|  |  |  | OUTPUT 3 |  |
|  |  |  | OUTPUT 4 |  |
|  |  | INPUT2 EDID | 1080P 2CH, Default |  |
|  |  |  | 1080P MLT |  |
|  |  |  | OUTPUT 1 |  |
|  |  |  | OUTPUT 2 |  |
|  |  |  | OUTPUT 3 |  |
|  |  |  | OUTPUT 4 |  |
|  |  | INPUT3 EDID | 1080P 2CH, Default |  |
|  |  |  | 1080P MLT |  |
|  |  |  | OUTPUT 1 |  |
|  |  |  | OUTPUT 2 |  |
|  |  |  | OUTPUT 3 |  |
|  |  |  | OUTPUT 4 |  |
|  |  | INPUT4 EDID | 1080P 2CH, Default |  |
|  |  |  | 1080P MLT |  |
|  |  |  | OUTPUT 1 |  |
|  |  |  | OUTPUT 2 |  |
|  |  |  | OUTPUT 3 |  |
|  |  |  | OUTPUT 4 |  |
|  |  | INPUT ALL | 1080P 2CH, Default |  |
|  |  |  | 1080P MLT |  |
|  |  |  | OUTPUT 1 |  |
|  |  |  | OUTPUT 2 |  |
|  |  |  | OUTPUT 3 |  |
|  |  |  | OUTPUT 4 |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | HDCP | HDCP | FOLLOW INPUT, Default |  |
|  |  |  | ALWAYS ON |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | AUDIO | MULTIVIEW MODE | OUTPUT1 AUDIO | IN 1, Default |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 2 AUDIO | IN 1 |
|  |  |  |  | IN 2, Default |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 3 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3, Default |
|  |  |  |  | IN 4 |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 4 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4, Default |
|  |  |  |  | OFF |
|  |  |  | BACK |  |
|  |  |  | EXIT |  |
|  |  | MATRIX MODE | OUTPUT1 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 2 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 3 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 4 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | BACK |  |
|  |  |  | EXIT |  |
|  |  | VIDEO WALL | OUTPUT 1 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 2 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 3 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | OUTPUT 4 AUDIO | IN 1 |
|  |  |  |  | IN 2 |
|  |  |  |  | IN 3 |
|  |  |  |  | IN 4 |
|  |  |  |  | FOLLOW, Default |
|  |  |  |  | OFF |
|  |  |  | BACK |  |
|  |  |  | EXIT |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | BACK |  |  |  |
|  | EXIT |  |  |  |
| NETWORK | IP ADDRESS | BYTE 1 | 192 |  |
|  |  | BYTE 2 | 168 |  |
|  |  | BYTE 3 | 0 |  |
|  |  | BYTE 4 | 200 |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | SUBNET MASK | BYTE 1 | 255 |  |
|  |  | BYTE 2 | 255 |  |
|  |  | BYTE 3 | 255 |  |
|  |  | BYTE 4 | 0 |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | GATEWAY IP | BYTE 1 | 192 |  |
|  |  | BYTE 2 | 168 |  |
|  |  | BYTE 3 | 0 |  |
|  |  | BYTE 4 | 1 |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | DHCP CLIENT | DHCP CLIENT | ON |  |
|  |  |  | OFF, Default |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | MEDIA TYPE | MEDIA TYPE | 10M |  |
|  |  |  | 100M |  |
|  |  |  | AUTO, Default |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | MAC FILTER | MAC FILTER | ON |  |
|  |  |  | OFF, Default |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | BACK |  |  |  |
|  | EXIT |  |  |  |
| SYSTEM | DEVICE ID | DEVICE ID | 0~99, Default 0 |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | REMOTE ID | REMOTE ID | 0~9, Default 0 |  |
|  |  | BACK |  |  |
|  |  | EXIT |  |  |
|  | FACTORY DEFAULT | FACTORY DEFAULT |  |  |
|  | REBOOT | REBOOT |  |  |
|  | INFORMATION | FIRMARE V1.00 |  |  |
|  |  | WEB V1.0 |  |  |
|  | BACK |  |  |  |
|  | EXIT |  |  |  |
| EXIT |  |  |  |  |

WebGUI Control:

Initial IP Configuration

- Setup host IP Address same as your PC with same subnet.(Default 192.168.0.200)

To change IP Address please enter OSD menu by panel button or IR remote control.

- Enter IP Address in your web browser and you could link to the web page of host.

- Browser support: Internet Explorer 9, Chrome 42.0.2311.135 m, Firefox 31.0 and higher

WebGUI Operating:

LOGIN:

- USERNAME : admin

- PASSWORD : 123

MAIN PAGE:

- ROUTING：Include MULTIVIEW, MATRIX, VIDEOWALL three mode

- SETUP：OUTPUT RESOLUTION, HDCP SETTING, ID SETTING, PICTURE SETTING, OSD SETTING

- NETWORK：IP ADDRESS, SUBNET MASK GATEWAY, DHCP CLIENT, MEDIA TYPE, MAC FILTER

- SYSTEM：change password

- ADVANCE：Adjust user define layout of MULTIVIEW mode

Multiview

- VIDEO SETTING：Select video source of window A~D to input 1~4 or off

- AUDIO SETTING：Select audio source of output 1~4 from input 1~4 or off

- WINDOW LAYOUT：Select windows layout from default 1~4 or user define 1~4

- MULTIVIEW SCENCE：Select SCENCE 1~8 to load or save

- REFRESH：Refresh the information of host

- APPLY：Apply changes

Matrix

- VIDEO SETTING：Select video source of output 1~4 to input 1~4, off or follow.

- AUDIO SETTING：Select audio source of output 1~4 from input 1~4,off or follow

- MATRIX SCENCE：Select SCENCE 1~8 to load or save

- REFRESH：Refresh the information of host

- APPLY：Apply changes

VIDEOWALL

- VIDEO SETTING：Display videowall block of output 1~4

- AUDIO SETTING：Select audio source of output 1~4 from input 1~4,off or follow

- Video SCENCE：Select SCENCE 1~4 to load

- REFRESH：Refresh the information of host

- APPLY：Apply changes

*SCENCE 1 is factory default

SETUP

- OUTPUT RESOLUTION：Support 480P, 576P, 720P50, 720P60, 1080P24, 1080P50, 1080P60,Default 1080P60, 1024x768, 1280x800, 1280x1024, 1366x768, 1440x900, 1600x1200, 1680x1050, 1920x1200

- HDCP SETTING：Set HDCP always on or follow input.

- ID SETTING：Set DEVICE ID, Default 0, REMOTE ID, Default 0

- PICTURE SETTING：Set CONTRAST,  BRIGHTNESS,  SATURATION,  HUE of output 1~4, Default 50

- OSD SETTING：Set MENU TIMEOUT, Default 20s,  INFO TIMEOUT, Default 5s, INFO POSITION, Default LEFT TOP,  INFO DISPLAY, Default ON

- REFRESH：Refresh the information of host

- APPLY：Apply changes

NETWORK

- IP ADDRESS：Set IP address, Default 192.168.0.200

- SUBNET MASK：Set Subnet mask, Default 255.255.255.0

- GATEWAY：Set gateway address, Default 192.168.0.1

- DHCP CLIENT：

ON:Get IP Address, Subnet Mask, Gateway from existing DHCP server

OFF:Set IP Address, Subnet Mask, Gateway by WebGUI or OSD menu manually, Default OFF

- MEDIA TYPE：Set link speed of Ethernet port to 10 / 100 / Auto detect, Default AUTO

- MAC FILTER: Add up to 10 registered MAC Address, only the MAC address which is in the list can be able to connect host by Ethernet, Default OFF.

- REFRESH：Refresh the information of host

- APPLY：Apply changes

SYSTEM

- PASSWORD：Enter old password, Default 123

- NEW PASSWORD：Enter new password

- CONFIRM PASSWORD：Enter new password again to confirm

- APPLY：Apply changes

ADVANCE

- WIDTH, HEIGHT: Set size of the windows.

- X.POS,Y.POS: Set top left position of window by x, y coordinate.

- PRIORITY: Set the priority of windows, 1 is top and 4 is bottom.

- BORDER: Select YES to show border of window, select NO to disable.

- COLOR: Set color of border.

- WINDOW LAYOUT：Select windows layout from user define 1~4 to load or save

- LOAD: Select windows layout from user define 1~4 to modify

- SAVE: Save windows layout to user define 1~4 after change the value

RS232 remote control command:

Connect the RS232 CONSOLE port to the COM port of the host by an F/M RS232 straight cable (pin 2 to pin 2; pin 3 to pin 3, pin 5 to pin 5)

Communication format is 9600, 8, N, 1 (9600 bps, 8 data bits, no parity check, 1 stop bit)

Operation Instruction format:

Command + △(Space) + Parameter + Enter

[Command]       			[Description]

-----------------------------------			-------------------------------------------

POWER				y			Power on/standby

SMOUT1				n			Output 1 from input n in seamless matrix

SMOUT2				n			Output 2 from input n in seamless matrix

SMOUT3				n			Output 3 from input n in seamless matrix

SMOUT4				n			Output 4 from input n in seamless matrix

SMOUTALL			n			All output from input n in seamless matrix

SMADOOUT1			n			Output 1 audio source from input n in seamless matrix

SMADOOUT2			n			Output 2 audio source from input n in seamless matrix

SMADOOUT3			n			Output 3 audio source from input n in seamless matrix

SMADOOUT4			n			Output 4 audio source from input n in seamless matrix

SMADOOUTAll			n			All output audio source from input n in seamless matrix

MVWINA				n			Window A from input n in multiview

MVWINB				n			Window B from input n in multiview

MVWINC				n			Window C from input n in multiview

MVWIND				n			Window D from input n in multiview

MVWINALL				n			Set all windows from input n in multiview

MVADOOUT1			n			Output 1 audio source from input n in multiview

MVADOOUT2			n			Output 2 audio source from input n in multiview

MVADOOUT3			n			Output 3 audio source from input n in multiview

MVADOOUT4			n			Output 4 audio source from input n in multiview

MVADOOUTALL			n			All output audio source from input n in multiview

VDWALLRX			n			Input port n in video wall

VDWALLADOOUT1		n			Output 1 audio source from input n in video wall

VDWALLADOOUT2		n			Output 2 audio source from input n in video wall

VDWALLADOOUT3		n			Output 3 audio source from input n in video wall

VDWALLADOOUT4		n			Output 4 audio source from input n in video wall

VDWALLADOOUTALL		n			All output audio source from input n in video wall

DHCP				y			DHCP Client on/off

IP_ADDRESS			[x.x.x.x]		Set IP address

SUBNET_MASK			[x.x.x.x]		Set subnet mask

GATEWAY				[x.x.x.x]		Set gateway

MEDIA_TYPE			z			Set Ethernet link speed

MAC FILTER			y			MAC filter on/off

VERSION							Display firmware version

HELP								Show all commands

[SYMBOL]							[PARAMETER]

--------------------------------------------------------------------------

N								0 | 1 | 2 | 3 | 4

n=0(OFF) n=1(INPUT1) n=2(INPUT2) n=3(INPUT3) n=4(INPUT4)

y								00=OFF, 01=ON

z								00=10M, 01=100M 02=AUTO

[x.x.x.x]							Ethernet parameter

EX.192.168.0.1

Example:

SMADOOUT1△03					Set output 1 switch to input 3 in MATRIX mode

MVADOOUT3△04					Set output 3 switch to input 4 in MULTIVIEW mode

VDWALLADOOUT3△02				Set audio output from input 2 in VIDEOWALL mode

Commands are case insensitive.

The host will reply OK once string is correct, NG in case of incorrect string

Specification:

| ITEM NO. | HM44 |
| --- | --- |
| Resolution | 480i/p, 576i/p, 720p, 1080i/p@60Hz |
| HDMI Input | HDMI Type A x 4 |
| HDMI Output | HDMI Type A x 4 |
| Ethernet Connector | RJ45 x 1 |
| RS232 Connector | DB9 Male x 1 (Console) |
| IR Carrier | ± 45 degree |
| Power Supply | DC 12V 4.16A |
| Power Consumption | 2A (Max) |
| Temperature | Operation: 0 to 55℃, Storage: -20 to 85℃, Humidity: up to 95% |
| Dimensions mm | 483 x 177 x 44 |
| Weight g | 2300 |

Rev. A V1.50 170718