HDMI to HD-SDI CONVERTER

ITEM NO.:  SDI02

SDI02 converts signal from HDMI to 3G-SDI, HD-SDI and SD-SDI which provides the highest quality images for professional audio-visual end users. Designed as a high performance video processor for post-production and broadcast applications, the HDMI to SDI Converter provides exceptional and enhanced image processing and quality.

Features:

- To send HDMI signal converter to 3G/HD/SDI signal

- Support video signal 480i, 576i, 720p, 1080i, 1080p.

- Support SDI (SMTP 259M), HD-SDI (SMTP 292M), 3G-SDI (SMTP 424M).

- Support audio LPCM 7.1 Channel.

Installation:

Panel View:

SDI02

Recommend Cable:

To send HD video must use coaxial cable with Bandwidth at least 1.5G , to send Full HD video must use coaxial cable with bandwidth at least 3G.

Link cable recommend to use high quality 75Ω Coaxial Cable & BNC connector , to use low quality coaxial cable may case signal loss and effect transmission distance and reliability.

Working Distance:

| Max. distance via high quality 75Ω Coaxial Cable ( Belden 1505A Testing) |  |
| --- | --- |
| SD (480i,576i resolution) (270Mbps) | 300M |
| HD (720p, 1080i, 1080p@24/25/30Hz resolution) (1.45Gbps) | 200M |
| Full HD (1080p@50/60Hz) resolution (2.97Gbps) | 100M |

LED Indication:

GREEN	         	    Power & Signal

ON 		    Power on

FLASH 		Not support HDMI input format

BLUE					SDI  signal

ON 			SDI  signal output

Switch Instruction:

1 		↑	Video  Support  480i, 576i, 720p@50/59.94/60Hz, 1080i@50/59.94/60Hz.

↓	Video  Support  480i, 576i, 720p@50/59.94/60Hz, 1080i@50/59.94/60Hz,

1080p@23.98/24/29.97/30/50/59.94/60Hz.

2		↑	Audio  Support  LPCM 2 Channels.

↓	Audio  Support  LPCM 8 Channels.

Trouble Shooting:

1. SDI signal not support HDMI 480p & 576p resolution.

2. SDI output resolution and frequency is same as HDMI input, if your SDI monitor not support HDMI

input resolution and frequency, then it won’t have picture at the monitor.

Specification:

| ITEM NO. | SDI02 |
| --- | --- |
| SDI Standard / Protocol | SDI (SMTP 259M), HD-SDI (SMTP 292M), 3G-SDI (SMTP 424M) |
| Resolution | NTSC (480i), PAL (576i), 720p@50/59.94/60Hz, 1080i@50/59.94/60Hz, 1080p@23.98/24/29.97/30/50/59.94/60Hz |
| Audio | LPCM 8 Channels, 48Khz, 16 / 20 / 24Bit |
| HDMI Output Connector | HDMI Type A |
| SDI Output Connector | BNC 75Ω |
| Power Consumption | 850mA (Max) |
| Power Supply | Switching DC5V 1AMP |
| Temperature | Operation: 0 to 55℃, Storage: -20 to 85℃, Humidity: up to 95% |
| Dimensions | 67 x 110 x 27 (mm) |
| Weight | 200 g |

Rev. B